## Typhoon Evacuation

Location:

Rong Doi Platform: 	Lat: 7.7925	Lon: 108.2021

BK-TNHA		Lat: 7.73	Lon: 107.93

- Introduction

Vietnam is situated in the path of Western North Pacific tropical storm and there is potential for severe weather which may threaten the personnel and facilities at Rong Doi field.

There will be more tropical cyclones formed in the tropical Western North Pacific than anywhere else in the world. More than 25 tropical storms develop each year and about 18 become typhoons. These typhoons are the largest and most intense tropical cyclones in the world.

Each year an average of five (5) generates maximum winds over 130 knots: Circulations covering more than 600 miles in diameter are not uncommon. Most of these storms from East of the Philippines and move across the pacific toward the Philippines, Japan, and China; only a few storms are formed in the Bien Dong Sea. The season extends from April through December.

However, tropical cyclones are more common in the off-season months in this area than anywhere else. The peak of the season is July through October, when nearly 70 percent of all typhoons develop. There is a noticeable seasonal shift in storm track in this region.

From July through October, storms move north of Philippines and re-curve, while early and late season typhoons move on a westerly track through the Philippines before re-curving.

- Purpose and Scope

The purpose of this procedure is to provide specific actions for responsible persons to take-in the event of a Tropical Revolving Storms (TRS) and in preparing Rong Doi Field for surviving any tropical storm.

The scope of this evacuation procedure is applied for Rong Doi PUQC and PPS 01 which is situated at Block 11-2 Southeast offshore Vietnam.

This section of procedure contains detailed plan to be adopted for evacuation of personnel from PUQC and PPS 01. The basic principle of this procedure should also apply to onshore support where in general the evacuation is more easily accomplished.

- Tropical Storm Classification

| Classification | Description | Parameter |
| --- | --- | --- |
| Class 1 | Tropical Disturbance | No revolving features. |
| Class 2 | Tropical Low or Depression | Start to revolving with wind speed less than 33 knots or force 6 and force 7 (Beaufort wind scale). |
| Class 3 | Tropical Storm | Revolving with wind speed between 34 to 47 knots or force 8 and force 9 (Beaufort wind scale). A tropical low that has developed to the point where 10 minutes sustained wind speed within the system are between 34 to 47 knots. |
| Class 4 | Severe Tropical Storm | Revolving with wind speed between 48 to 63 knots or force 10 and force 11 (Beaufort wind scale-please refer to appendix 2). A tropical low that has developed to the point where 10 minutes sustained wind speed within the system are between 48 to 63 knots. |
| Class 5 | Typhoon | Revolving with wind speed between 64 to 126 knots or force 12 (Beaufort wind scale-please refer to appendix 2).A tropical low that has developed to the point where 10 minutes sustained wind speed within the system are between 64 to 126 knots. |
| Class 6 | Super Typhoon | Revolving with wind speed greater than 127 knots or above force 12 (Beaufort wind scale-please refer to appendix 2). A tropical super typhoon in which 10 minutes sustained wind speed within the system exceed 127 knots. |

- Weather Forecasting Services

Weather forecasts are received from the contracted weather service provider.

In addition, weather information can be received via dedicated Typhoon Warning Centre in Australia, Hong Kong, Tokyo and Navtex as well as some weather web site at Appendix

Reporting Frequency: The contracted weather service provider will provide 72 hours weather forecast to ZNEPV as below:

Routine Range Forecast		- Updated every 12 hours

Storm Surveillance Forecast	- Updated every 6 hours

Declaration of Storm or Typhoon	- Updated every 3 hourly to 2 hourly or upon the ZNEPV request.

The weather information provided is used by DIM and Marine Advisor to track the path of approaching severe weather in order for appropriate actions to be taken to enable an orderly evacuation.

- ‘STORM ALERT’ Levels

- To ensure all concerned personnel are kept informed and timely response to severe typhoon, a three color coding system for ‘STORM ALERT’ is recommended.

- The alert is signaled when the ‘STORM EDGE’ is within the radius distances from sites. Each condition of ‘STORM ALERT’ is indicated by a color, i.e. GREEN, YELLOW and RED as shown below:

| Alert Color | Action | Typhoon Position |
| --- | --- | --- |
| GREEN ALERT | -Alert all units/personnel in path of typhoon. -Gather information and identified action plan. -Monitor progress of typhoon. -Request weather forecast at 3 hourly intervals. -Supervisory judgment depending on current weather conditions will be made as to whether maintenance of operations continued or not. | GREEN ALERT signals mean that the edge of a Tropical Disturbance or Depression that could develop into a ‘STORM’ is within 750nM of the platform. GREEN ALERT status does not have any impact on normal offshore operations; however it initiates tightened safety measure at offshore. |
| YELLOW ALERT | -Make preparations and secure all operations and loose items. -Make preparation to evacuate of non-essential personnel. -Only a single helicopter load of essential personnel to remain offshore by the time typhoon reaches outer boundary of Red Zone. -All standby vessels to have reached safe area by the time the typhoon reaches the outer boundary of the Red Zone. -Commence evacuation of all remaining personnel in sufficient time to reach safe locations by the time the typhoon crosses the outer boundary of the Red Zone. | YELLOW ALERT signals mean- triggers preparation for abandonment of platform and thus the work in progress must be immediately completed and secured. A YELLOW ALERT shall be declared by the OIM/FM when conditions fall within the calculated YELLOW ALERT radius. OR a TRS develops is within 750nM to 240nM from platform or 36hrs distance of typhoon traveling speed from facilities or whichever is least. |
| RED ALERT | -Confirm location of all essential personnel and evacuate to safe location. -Confirm location of all vessels. -Confirm status of all operations. | RED ALERT signals mean-triggers the final phase of the TRS procedure and platform abandonment. A RED ALERT shall be declared by the OIM/FM when conditions fall within the calculated RED ALERT radius. OR a TRS develops is less than 240nM from platform or less than 24hrs distance of typhoon traveling speed from facilities or whichever is least. |

- The radiuses for the YELLOW ALERT and RED ALERT are based on calculation which consider operational time constrain and ‘STORM’ movement speed.

- The ‘STORM ALERT’ levels conditions serve as GUIDELINES ONLY. The boundaries of these phases are dependent on the intensity, approach and 	velocity of the typhoon. The YELLOW Radius and RED Radius boundary can be determined by applying the formula below:

- RED Radius. The RED ALERT radius is calculated using the following formula:

‘Rr’ = Rt x Sm

Where:

‘Rr’ is “RED radius”

‘Rt is “RESPONSE time”. Is the time window required for the facilities to complete its STORM response?

‘Sm’ is “STORM movement”. Is the latest forecast from the MET office of the speed of advances of the STORM eye?

- For Example:

| Typhoon Velocity | Max Wind | 50 knots radius(R) |
| --- | --- | --- |
| 10 knots | 65 knots | 45nM |

A RED radius calculation base on a storm eye moving at ‘Sm’=10 knots and the response time ‘Rt’= 24 hours.

‘Rr’ = 24 hrs x 10 knots

‘Rr’ = 240nM to the EDGE of the STORM.

- YELLOW Radius. The YELLOW ALERT signal stage is based on a YELLOW radius reaching the site. The YELLOW radius is calculated using the following formula:

‘Yr’= Rr + (Sot x Sm)

Where:

‘Yr’ is “YELLOW radius”

‘Rr’ is “RED radius”

‘Sot’ is “Secure operation time”

‘Sm’ is “STORM movement”

- For Example:

| Typhoon Velocity | Max Wind | 50 knots radius(R) |
| --- | --- | --- |
| 10 knots | 65 knots | 45 nM |

Rr = 240nM (from RED radius)

Sot = 12 hours

Yr = 240nM + (12 hrs x 10 Knots)

Yr = 240nM + 120nM

Yr = 360nM to the EDGE of STORM

- Storm Alert Procedures

  - ‘GREEN ALERT’

- A ‘GREEN ALERT’ signal shall be declared if a typhoon or potential typhoon threat, which exists in the Nam Con Son Basin and could affect Rong Doi and PPS 01, has its center located within 750nM of the facilities or within 48hrs from the facilities.

- Action to be implemented on receipt of weather report that the typhoon is entering or about to enter the ‘GREEN ALERT’ phase is as below:

| Action Party | Action |
| --- | --- |
| OIM/FM | - Declare a ‘GREEN ALERT’ signal as per ‘STORM ALERT’ levels in section 8.0. - Inform DIM and advice the current operation status. - Inform VNHS service to be on standby and “Ready to take off” status. - Notify all POB of the ‘GREEN ALERT’ condition via the PA system. - Ensure all emergency communications are operational. - Inform ERT Leader and brief him on the situation. - Notify and liaise with FM PPS 01 as appropriate on information flow regarding weather condition. - Liaise with HSSE Manager with regard to this Procedure. - Monitor typhoon path and strength every 3 hourly and regularly liaise with Telecom Technician. - Start plotting and tracking the typhoon path on the ‘Storm Tracking Sheet’ chart-see Appendix 3. - Identify actions required to make the operations safe and all loose equipment is secured. - Review activities and minimize outside activity-keep personnel inside as necessary or suspension of non-routine activities on PUQC and WHP. - Identify safe locations for all personnel. - Plan, identify and list down non-essential personnel for the evacuation. - Identify minimum number of essential personnel to make operation safe. - Prepare plan for final evacuation as appropriate. - Identify marine craft under your control. Inform them of weather situation and their action and time required to reach a safe place. - Ready to sail “In Port” of a spot chartered supply vessel if any. - Apart from the above actions, normal operation in the field will be maintained. |
| OSC/ HSE Advisor | - Mobilize ERT members and brief them on the situation. - Identify all affected assets in the path of the typhoon. - Identify a safe location and method of transport for the evacuation of all personnel. - Prepare evacuation list and time to start. - Ensure that minimal risk to personnel is maintained. |
| Telecom Technician | - Monitor typhoon path and strength-record weather conditions on a 3 hourly and report to OIM/FM. - Liaise with other facilities and FM PPS 01 to maintain information flow regarding weather conditions. |
| CCR | - Review SD and depressurize procedure for process and WHP. - Assist OIM/FM and supervisor as and when required. |
| Medic | - Ensure medical supplies and stretcher is kept in a state of readiness. - Secure the medical room. - Liaise with OIM/FM to identify non-essential personnel for initial evacuation. |
| DIM | - Request up-to-date information from the OIM/FM. - Inform the BST of the typhoon threat and action plan. - Monitor the situation. - Inform HSSE Manager to be on standby to mobilize IMT members. - Appoint an IMT member to monitor weather forecast and track the typhoon on the typhoon map. - Appoint an IMT member to identify and reserve helicopter and road transport requirement. - Identify a safe place for all personnel. - Appoint an IMT member to record key action items from time to time. - Regularly contact with the OIM/FM for up-to-date information. |
| Vung Tau Supply Base Logistics Officer | - In consultation with the DIM prepare a logistic plan for the evacuation of PUQC and PPS 01. - Ensure helicopter contractor is alerted and the crew/helicopter is on standby. - Obtain last POB list and check names of persons as they arrive at arrival point. - Organize reception of evacuees at arrival point and ensure availability of refreshment, transport, accommodation etc. - Liaise with contractors employers to keep them informed on latest situation. |
| All POB | - Check their work place and secure any loose items. - Remain alert and follow the OIM/FM instructions. |

  - ‘YELLOW ALERT’

- A ‘YELLOW ALERT’ signal shall be declared when the path and strength of a typhoon has its center located within the calculated yellow alert radius or between 750nm to 240nm of Rong Doi and PPS 01 or within 36hrs from the facilities.

- Action to be implemented on receipt of weather report that the typhoon is entering or about to enter the ‘YELLOW ALERT’ levels is as below:

| Action Party | Action |
| --- | --- |
| OIM/FM | - Declare ‘YELLOW ALERT’ signal as per ‘STORM ALERT’ levels in section 8.0. - Inform ERT Leader to respond as per Typhoon Evacuation Procedure. - Contact OSC, confirm situation and advise him that you are implementing agreed evacuation plan. - Suspend work on platform. - Immediately commence evacuating non-essential personnel to agreed safe place. - Continue to monitor the track of typhoon path and strength every hourly. - Maintain frequent contact with the Telecom technician and DIM. - Termination of an export operation and PPS 01 release of the export tanker if any. - Secure deck cargo and continue back loading all vessels if possible - Ensure transport and escape route are available for the remaining personnel. |
| OSC/ HSE Advisor | - Re-call all ERT members, brief them on the latest situation and that the OIM/FM is implementing agreed evacuation plan. - Instruct ERT member to secure operations and commence securing loose equipment. - Assist in the evacuating non-essential personnel. - Confirm accommodation in the safe place is made ready to receive evacuees. - Continue to track and take care of evacuees until returned to place of work or home. - Maintained a timed log of all actions. - Continue to monitor the track of typhoon and maintain frequent contact with the OIM/FM. |
| Telecom Technician | - Monitor typhoon path and strength-record weather conditions on a 2 hourly and report to OIM/FM. - Liaise with other facilities and PPS 01 to maintain information flow regarding weather conditions. |
| CCR | - Review SD and depressurize procedure for process and WHP. - Assist OIM/FM and supervisor as and when required. |
| Medic | - Liaise with OIM/FM and assist with evacuation of non-essential personnel. |
| DIM | - Obtain up-to-date information from the OIM/FM. - Inform the BST of the typhoon threat and evacuation plan. - Instruct appointed person to inform all relevant Government Agencies. - Provide advance notice to NCSP of possible platform Shut Down. |
| Vung Tau Supply Base Logistics Officer | - Monitor helicopter flight and availability. - Inform OIM/FM and OSC on admin and logistic preparation at arrival point. - Keep DIM informed on latest status. - Confirm accommodations, transportation and refreshment for evacuees. |
| All POB | - Follow instruction as directed by OIM/FM and OSC. - Cease all work and return PTW to the CCR. - Check work place and secure all loose items. - Stay in accommodations except for essential activities as directed by OIM/FM. |

  - ‘RED ALERT’

- A ‘RED ALERT’ signal will be issued when the path and strength of typhoon has its centers located within the calculated red alert radius or 240nm of the Rong Doi and PPS 01 or within 24hrs from the facilities.

- Actions to be completed on receipt of weather information that the typhoon has crossed the outer boundary of the ‘RED ALERT’ levels as below:

| Action Party | Action |
| --- | --- |
| OIM/FM | - Declare ‘RED ALERT’ signal as per ‘STORM ALERT’ levels in section 8.0. - Inform OSC to respond as per this Procedure. - Confirm with ERT leader that the facilities have been secured to withstand the typhoon. - Rapid completion of all outstanding work. - Immediately evacuate last remaining essential personnel to agreed safe location. - Maintain communication with DIM and advice of evacuation progress. |
| OSC/ HSE Advisor | - Re-call all ERT members, brief them on the latest situation that the typhoon is entering the Red Zone. - Confirm that the facilities are all secured and in safe condition. - Instruct all remaining essential personnel to evacuate as directed by the OIM/FM. - Record report of damage and continue to maintain a timed log of all actions. - Inform and brief OIM/FM on latest issue and activities. |
| Telecom Technician | - Monitor typhoon path and strength-record weather conditions on a 1 hourly and report to OIM/FM. - Liaise with other facilities and PPS 01 to maintain information flow regarding weather conditions. - Coordinate air traffic as required for the remaining personnel. - Will prepare to abandon the platform. |
| CCR | - Upon instruction from the OIM/FM-SD and de-pressurize production facilities. - Assist the OIM/FM and Production supervisor as required. - Will prepare to abandon the platform. |
| Medic | - Secure all medical facilities. - Ensure drugs cabinet is locked prior to abandon platform. - Will prepare to abandon the platform. |
| DIM | - Obtain up-to-date information from the OIM/FM and maintain contact with him. - Inform the BST regarding the typhoon and situation of personnel. - Instruct appointed person to inform all relevant Government Agencies. - Issue a written report to PVN and ZNEPV head office. - Notify NCSP of platform shut down and evacuation. |
| Vung Tau Supply Base Logistics Officer | - Monitor helicopter schedule and movement. - Allocate transportation, refreshment and accommodation for personnel upon arrival. - Keep DIM informed and assist OIM/FM. |
| All POB | - Prepare to SD facilities as directed by OIM/FM. - Prepare for evacuation as directed by OIM/FM. |

- Method of Evacuation

  - By Helicopter

- In normal circumstances evacuation of all non-essential personnel and essential personnel are by helicopter as per prepared plan.

- One helicopter shall be on standby to take off at any times.

- Day time transfer is always preferable in marginal weather conditions.

- 2nd helicopter ready for take-off within 2 hours’ notice.

- In unforeseen circumstances evacuation by vessel may be considered to the nearest temporary safe location i.e. Con Dao Island.

  - By Standby Vessel or Supply Vessel

- Evacuation by Standby Vessel or Supply Vessel to be taken if evacuation by helicopter is not feasible.

- Marine Specialist shall ensure that Standby Vessel or Supply Vessel safety equipment and towing equipment shall be made available to perform specific duties such as evacuation duties during the forthcoming typhoon season.

- In case of immediate ‘Typhoon RED ALERT’ being declared by OIM/FM all Supply Vessels arriving at the field shall be mobilized to evacuate all personnel to temporary safe location.

- When using a Personnel Transfer Basket particular attention must be considered to the relative motion between the crane and Standby Vessel or Supply Vessel.

- All Supply Vessels arriving at the fields during typhoon season shall offload cargo as soon as possible, in order to clear deck for evacuation task if necessary.

- Based on weather forecast, evacuation of all non-essential personnel by vessel to Con Dao Island can be considered if there is no helicopter.

- PUQC to Con Dao Island (alternative/temporary safe location) will take at least 10 hours by vessel.

- Standby Vessels shall have the maximum amount of fuel, food and water at secure levels for a minimum 15 days of operations during typhoon season.

- Any spot chartered Supply Vessel at VT if required to assist the evacuation in the field will sail immediately.

- Re-manning of the PUQC and PPS 01 after Typhoon

- Order of re-manning the facilities shall be decided by the both OIM/FM PPS 01 in the consultation with DIM.

- A details inspection of the facilities shall be made by the both OIM/FM PPS 01 and his supporting teams before the full POB are mobilized.

- Any adverse findings shall be reported to BST for immediate action and follow up.

- OIM (PUQC) and FM PPS 01 will complete the full damage report within one week and submit to ZNEPV operation teams for further action and lessons learned.

- Record

All report and records will be maintained by OIM/FM and a copy will be sent to Operation Team and keep at ZNEPV Ho Chi Minh Office.

- Information Sources for Weather Sites-Web pages.

- http://metocph.nmci.navy.mil/jtwc/warnings/wp2107

- http://www.jma.go.jp/en/typh/

- http://www.weather.gov.hk/wxinfo/currwx/tc.pos.htm

- http://www.ffwc.net/

- http://www.npmoc.navy.mil/

- http://www.yoko.npmoc.navy.mil/

- www.supertyphoon.com/index.html

- www.npmoc.navy.mil/jtwc.html

Figure 1: Beaufort Wind Scale.

| Beaufort number | Wind speed |  |  |  | Mean wind speed (kt / km/h / mph) | Description | Wave height |  | Sea conditions | Land conditions | Sea state photo |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | kt | km/h | mph | m/s |  |  | m | ft |  |  |  |
| 0 | 0 | 0 | 0 | 0-0.2 | 0 / 0 / 0 | Calm | 0 | 0 | Flat. | Calm. Smoke rises vertically. |  |
| 1 | 1-3 | 1-6 | 1-3 | 0.3-1.5 | 2 / 4 / 2 | Light air | 0.1 | 0.33 | Ripples without crests. | Wind motion visible in smoke. |  |
| 2 | 4-6 | 7-11 | 4-7 | 1.6-3.3 | 5 / 9 / 6 | Light breeze | 0.2 | 0.66 | Small wavelets. Crests of glassy appearance, not breaking | Wind felt on exposed skin. Leaves rustle. |  |
| 3 | 7-10 | 12-19 | 8-12 | 3.4-5.4 | 9 / 17 / 11 | Gentle breeze | 0.6 | 2 | Large wavelets. Crests begin to break; scattered whitecaps | Leaves and smaller twigs in constant motion. |  |
| 4 | 11-15 | 20-29 | 13-18 | 5.5-7.9 | 13 / 24 / 15 | Moderate breeze | 1 | 3.3 | Small waves. | Dust and loose paper raised. Small branches begin to move. |  |
| 5 | 16-21 | 30-39 | 19-24 | 8.0-10.7 | 19 / 35 / 22 | Fresh breeze | 2 | 6.6 | Moderate (1.2 m) longer waves. Some foam and spray. | Smaller trees sway. |  |
| 6 | 22-27 | 40-50 | 25-31 | 10.8-13.8 | 24 / 44 / 27 | Strong breeze | 3 | 9.9 | Large waves with foam crests and some spray. | Large branches in motion. Whistling heard in overhead wires. Umbrella use becomes difficult. |  |
| 7 | 28-33 | 51-62 | 32-38 | 13.9-17.1 | 30 / 56 / 35 | Moderate gale | 4 | 13.1 | Sea heaps up and foam begins to streak. | Whole trees in motion. Effort needed to walk against the wind. |  |
| 8 | 34-40 | 63-75 | 39-46 | 17.2-20.7 | 37 / 68 / 42 | Fresh Gale | 5.5 | 18 | Moderately high waves with breaking crests forming spindrift. Streaks of foam. | Twigs broken from trees. Cars veer on road. |  |
| 9 | 41-47 | 76-87 | 47-54 | 20.8-24.4 | 44 / 81 / 50 | Strong Gale | 7 | 23 | High waves (6-7 m) with dense foam. Wave crests start to roll over. Considerable spray. | Light structure damage. |  |
| 10 | 48-55 | 88-102 | 55-63 | 24.5-28.4 | 52 / 96 / 60 | Storm | 9 | 29.5 | Very high waves. The sea surface is white and there is considerable tumbling. Visibility is reduced. | Trees uprooted. Considerable structural damage. |  |
| 11 | 56-63 | 103-119 | 64-73 | 28.5-32.6 | 60 / 112 / 70 | Violent storm | 11.5 | 37.7 | Exceptionally high waves. | Widespread structural damage. |  |
| 12 | 64-80 | 120 | 74-95 | 32.7-40.8 | 73 / 148 / 90 | Hurricane | 14+ | 46+ | Huge waves. Air filled with foam and spray. Sea completely white with driving spray. Visibility greatly reduced. | Considerable and widespread damage to structures. |  |

- Form

STORM TRACKING SHEET

| Report Time (Date + Hours) | Period since last report (in Hours) (2) | Radius of Storm (nm) (3) | Position of Storm Eye |  |  | Speed of Eye (nm/hrs) (7)=(6)/(2) | Dist to Site (nm) (8) | Time to Reach Site (Hours) (9)=(8)-(3)/(7) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (1) |  |  | Longitude (Easting) (4) | Latitude (Northing) (5) | Dist. From Prev (nm) (6) |  |  |  |  |

Note:

1. From the weather report, fill in columns (1), (3), (4) and (5)

2. Subtract current report time from previous report time in column 1 and enter into column 2

3. Measure the points plotted on the scaled map for column 6 and 8

4. Calculation for column (7) as shown

5. Calculation for column (9) as shown

- Tropical Depression /Typhoon: Immediate Actions Check sheet

GREEN ALERT ..

A Green Alert will be issued that a typhoon or potential typhoon threat exists in the Nam Com Son Basin and could affect the Rong Doi Platform within 48 hours or is within 750 NM from the field.

| Emergency Response Procedures |  |
| --- | --- |
| Telecom Technician | Check |
| Monitor typhoons path and strength - record weather indicators on a 3 hourly basis and report to IM (OIM) |  |
| Liaise with other facilities in the area to maintain information flow regarding weather conditions |  |
| On Duty CRT | Check |
| Review shutdown and depressurizing procedure for Process and WHP |  |
| Assist IM (OIM) and Supervisor as requested |  |
| Offshore Installation Manager (IM) | Check |
| Declare a blue alert and notify DIM onshore. |  |
| Inform onshore management and air operations to be on standby and all POB of the typhoon threat. |  |
| Ensure all emergency communications are operational. |  |
| Production shutdown procedures are reviewed and ready |  |
| Helideck safe for helicopter landing and refueling operations. |  |
| Monitor typhoon path and strength 3 hourly - liaise / confirm with Telecom Technician. |  |
| Ensure all loose plant & equipment is secured in preparation for high winds / high sea state conditions. |  |
| Inform FSO, supply vessels within the field and Shore base of Blue alert and forward plan for possible evacuation. Shutdown off-take operations as appropriate. |  |
| Ensure helicopter services are aware of weather conditions and advise of possible non-essential personnel evacuation - inform no. of PAX deemed to be Non-essential personnel. |  |
| Review activities and minimize outside activity – keep personnel inside as necessary. |  |
| Prepare a plan for personnel to remain onboard in the event that evacuation is not attainable for other reasons ( e.g. helicopters may be grounded ) |  |
| Emergency Response Teams / Medic | Check |
| Secure Medical room, ensure Medical supplies/ stretchers are kept in a state of readiness. |  |
| Liaise with OIM to confirm personnel identified as non-essential for initial evacuation as required. |  |
| DIM | Check |
| To initiate onshore emergency response, contacting the IMT and helicopter contractor. |  |
| Confirm number of POB offshore and those deemed to be non-essential in case of evacuation operations as per Yellow alert. |  |
| Establish evacuation plan in conjunction with OIM / Facility Manager for both Platform and FSO within the Rong Doi Field. |  |

- Tropical Depression /Typhoon: Immediate Actions Checksheet

YELLOW ALERT …

A Yellow Alert will be issued when the path and strength of typhoon has been established and may affect the Rong Doi within 36 hours or is within 750 - 240 nM of the field.

| Emergency Response Procedures |  |
| --- | --- |
| Telecom Technician | Check |
| Monitor typhoons path and strength - record weather indicators on a 1 hourly basis and report to IM (OIM) |  |
| Liaise with other facilities in the area to maintain information flow regarding weather conditions |  |
| On Duty CRT | Check |
| Review shutdown and depressurizing procedure for Process and WHP. Preparation for shutdown as required |  |
| Assist IM (OIM) and Supervisor as requested |  |
| Offshore Installation Manager (IM) | Check |
| Declare a yellow alert |  |
| Start evacuation of non-essential personnel and coordinate with FSO Supt. |  |
| Inform onshore management and request air operations to start evacuation |  |
| Ensure all emergency communications are operational |  |
| Production shutdown / depressurization procedures are reviewed and ready |  |
| Helideck safe for helicopter landing and refueling |  |
| Monitor typhoon path and strength hourly in conjunction with Telecom Technician. |  |
| Confirm with FSO that all offtake operations have ceased |  |
| Liaise with Supply boats – demob vessel and non-essential personnel on vessel to safe haven if deemed necessary |  |
| Determine the need for early evacuation taking into consideration helicopter limitations and the number of persons that need to be evacuated from the facility and adjoining FSO. |  |
| Ensure Nav –aids and Non directional beacon are operational |  |
| Confirm preparations are underway for total evacuation of facility |  |
| Emergency Response Teams / Medic | Check |
| Assist with evacuation on non-essential personnel as required |  |
| DIM | Check |
| To initiate onshore emergency response, contacting IMT and helicopter contractor. |  |
| Arrange accommodation for personnel coming onshore. |  |

- Tropical Depression /Typhoon: Immediate Actions Checksheet

RED ALERT..

A Red Alert will be issued when the path and strength of typhoon has been established and will affect the Rong Doi field within 24 hours or is less than 240 NM from the field.

| Emergency Response Procedures |  |
| --- | --- |
| Telecom Technician | Check |
| Will prepare to abandon the platform |  |
| Co-ordinate air traffic as required for necessary air evacuation operation of remaining personnel. |  |
| Liaise with other vessels/FSO within the field and advise that abandonment operations are underway. |  |
| Monitor typhoons path and strength - record weather indicators on a 1 hourly basis and report to IM (OIM). |  |
| Liaise with other facilities in the area to maintain information flow regarding weather conditions. |  |
| On Duty CRT | Check |
| Upon instruction from the IM (OIM) or his designate - shutdown and de-pressure production facilities. (ESD-1) Ensure the Well SCSSVs are in the closed position |  |
| Assist the Operations Supv. / IM (OIM) as requested |  |
| Prepare to abandon the platform |  |
| Offshore Installation Manager (IM) | Check |
| Declare a Red Alert situation to all POB. |  |
| Ensure emergency alarm procedures followed, muster and account for POB |  |
| Ensure all personnel remain inside the accommodation except for deemed essential services such as preparing to abandon the platform activities. If outside activities are necessary, ensure 2 personnel are deployed to actively assist and look out for each other. Record all outside personnel movements via Muster controller. |  |
| Inform FSO and other field support vessels / services as to Red Alert status. |  |
| Commence to evacuate personnel as required from the Facility utilizing air or marine support as required / available. |  |
| Confirm the Nav-aids are operational prior to abandonment. |  |
| Prior to abandonment – it is not advisable to leave any motor driven equipment running such as Emergency Generators or Fire pumps except for emergency reasons. |  |
| Emergency Response Teams / Medic | Check |
| Secure all medical facilities – ensure drugs cabinet is locked prior to departure of platform. |  |
| DIM | Check |
| To initiate onshore emergency response, contacting the IMT and helicopter contractor. |  |

Mobilization of Personnel after the Typhoon passes following evacuation.

The first essential crews to the facility should only be mobilized after the Typhoon has passed clear of the Rong Doi field and the wind speed and weather conditions are suitable for helicopter operations at the platform.

Closely monitor the typhoon movement and strength so that any standby vessels / tenders can be advised to return to the field. This may be prior to helicopter operations re-commencing to the platform.

Priority of personnel returning to the Rong Doi field is at the discretion of the DIM and the IM (OIM).